Mass cytometry for detection of silver at the bacterial single cell level

Yuting Guo1, Sabine Baumgart2, Hans-Joachim Stärk3, Hauke Harms1, Susann Müller1*

1Department of Environmental Microbiology, Helmholtz Centre for Environmental Research, Leipzig, Germany

2Department of Immune Monitoring, German Rheumatism Research Centre, Berlin, An-Institute of the Leibniz Association, Germany

3Department of Analytical Chemistry, Helmholtz Centre for Environmental Research, Leipzig, Germany

*Correspondence to: Susann Müller, E-mail: susann.mueller@ufz.de

Table of Contents

Table S1. Medium components for P. putida KT2440 cultivation	2

Figure S1. Calibration curve between cell counts and OD600	2

Figure S2. Cell indicator for CyTOF: ruthenium red and cisplatin staining optimization	2

Figure S3. CyTOF analysis plots	3

# Table S1. Medium components for P. putida KT2440 cultivation

|  |  | Component | g/L |
| --- | --- | --- | --- |
|  | Minerals | (NH4)2SO4 | 2.2 |
|  |  | MgSO4×7H2O | 0.4 |
|  |  | KH2PO4 | 2 |
|  | Trace elements | Na-citrate | 0.015 |
|  |  | FeSO4×7H2O | 0.01 |
|  | Carbon source | Glucose | 1 |

PBS composition: Na2HPO4 0.85 g/L, NaH2PO4 0.22 g/L, NaCl 8.5 g/L, pH 7.2.

# Figure S1. Calibration curve between cell counts and OD600

# Figure S2. Cell indicator for CyTOF: ruthenium red and cisplatin staining optimization

To optimize RR staining fixed P. putida KT2440 cells (in 2 % PFA 30 min, washed and re-suspended in 70 % ethanol) were used from a stationary phase sample.  The cells were treated with a 4°C stock solution of 1.3 µM RR in PBS. 0 to 0.33 µM of RR was tested on 108 cells / 200 µL PBS for both 30 and 60 min. The final staining protocol requires 0.13 µM RR / 108 cells / 200 µL PBS and a staining time of 30 min.

To optimize cisPt staining a mixture of 50 % live and 50 % dead cells of a stationary phase P. putida KT2440 sample was used. The cells were treated with a -20°C stock solution of 25 mM cisPt. 0 to 20 µM of cisPt were tested on 108 cells / 1 mL PBS and treated for 1 to 30 min. Following, the cells were treated with RR according to the optimized method described above. The final staining protocol requires 5 µM cisPt / 108 cells / 1 mL PBS and a staining time of 10 min.

102Ru

195Pt

104Ru

102Ru

A

B

A calibration curve based on RR and cisPt measurements

Different ratios of live and 70 % ethanol treated dead cells (20 min, room temperature) were prepared, stained by cisPt+RR, and measured at CyTOF.  In parallel the same ratios of cells were analyzed by using PI and flow cytometry. Standard errors were from replicate analysis.

C

# Figure S3. CyTOF analysis plots

P. putida KT2440 cells were cultivated in M12 medium and treated with AgNP-10 (1.29 mg/L). Examples showed cells harvested at 12 h and 72 h, and analyzed by CyTOF. From left to right, first column: all cell events were marked based on 102Ru/104Ru intensity; second column: the live/dead cells were discriminated based on 195Pt intensity; third column: the counts of dead cells containing silver, forth column: the counts of live cells containing silver. The related intensity of silver (IAg, median) in cisPt+dead and cisPt-live cells was shown in Table 1.

102Ru

102Ru

102Ru

102Ru

107Ag

107Ag

107Ag

Dead

Live

Dead

Live

107Ag subset:

Mean: 22.5

Median: 18.1

107Ag subset: Mean: 24.0

Median: 19.9

107Ag subset: Mean: 14.1

Median: 12.0

107Ag subset: Mean: 5.33

Median: 3.08

12 h

AgNP-10

72 h

AgNP-10

Counts

Counts

Counts

Counts

104Ru

104Ru

195Pt

195Pt

Dead

Live